ANX7440/30/90/96 layout Checklist:

| 1 | [Elective] Placement is important, please try to put ANX7440/30/90/96 to wide and open area. |  |
| --- | --- | --- |
| 2 | [Mandatory] It’s important to have a complete VGND plane underneath, and, - VGND shall be out of the chip package. It is only for DC/DC circuits. - VGND connect to GND at a single point which should be remote to ANX74xx and clean. - High speed traces should reference to GND and keep distance with VGND. EL: ok | Good Example: |
| 3 | [Mandatory] Put 2 via when VGND fanout to inner layer EL: please add one more via for VGND. | Example |
| 4 | [Mandatory] Put all DC/DC components on VGND EL:ok | Example Layout/schematic |
| 5 | [Mandatory] DC/DC input decouple capacitor location, should be close to VIN pin EL:ok | Good Example |
| 6 | [Mandatory] 1). Trace to VIN should be wide, shall fan-out as 8 mil wide trace. Increase width space available. 2). Fanout DC/DC VX output pin at 10mil width trace and widen it to inductor. Wider trace is better. WL:1 ok. 2 ok. |  |
| 7 | [Mandatory] Add a complete AVDD09 plane on layer VCC. EL:ok | Example layer E.g. Create a AVDD09 in layer 4, also see DVDD09, VDD18 |
| 8 | [Mandatory] Type C side AVDD09 pin has decouple capacitor (0.01uF) nearby. And, try to put all AVDD09 decouple capacitors near to pin(ANX7440). EL:ok |  |
| 9 | [Mandatory] Length match rule: Differential pair P/N trace length match. (Ideally, length difference equal to zero) - Length match to every section of the trace pair. E.g. pin to DC block capacitor, DC block capacitor to connector pin. EL: please provide length table. |  |
| 10 | [Mandatory] Length match rule: Inter-pair skew control, pair to pair length match with-in 500mil. [Elective] Length match rule: Inter-pair skew control, pair to pair length match with-in 50mil, for best margin without crowed routing, e.g. dense snake trace. EL: please provide length table. |  |
| 11 | [Mandatory] Spacing rule: High-speed differential trace must keep 3xW (which is center to center distance) spacing to other net. Snake trace should also keep 3xW distance to itself. E.g. 3.5/3.5/3.5 trace should be around 3x7= 21 mil spacing to other net. EL:ok |  |
| 12 | [Mandatory] Spacing rule: - XTAL and other noise component should keep distance from high speed lane as far as possible. Minim space is 3xW - The DC/DC inductor should be shielded component. If not, it should be 100mil away from the ANX7440 - Keep high speed signal off DC/DC.100Mil clearance is best. EL: Keep high speed signal off DC/DC.100Mil clearance is best. |  |
| 13 | [Mandatory] Do not route power/high speed traces inside package area EL: if possible, next time, remove SDA/SCL from inside package area. | Example: |
| 14 | [Mandatory] Remove GND under USB/DP high speed decouple capacitors/resisters/ESDs, if their pad width is greater than 2xtrace width. Acceptable void could be as right. EL: Remove GND under USB/DP high speed decouple capacitors/resisters/ESDs. | Example: |
| 15 | [Mandatory] High speed differential trace impedance must keep along the trace. Reference plane must keep to GND. DP/USB traces must center at differential 90 ohm |  |
| 16 | [Mandatory] Max via number along a High-speed USB/DP trace are two. EL:ok |  |
| 17 | [Mandatory] SDA/SDC signal routing in pair EL: please follow up next revision. |  |
| 18 | [Mandatory] DP AUX signal routing in pair EL: please modify in next revision |  |
| 19 | [Mandatory] High speed trace keep distance to reference plane edge, it requires 4xW (which is center to center distance) E.g. 3.5/3.5/3.5 trace should be around 4x7= 28 mil spacing to edge of reference plane The minim spacing is 3xW EL:ok | Good example |
| 20 | [Mandatory] Stitching ground via is required for weak coupling differential vias that could not provide ideal return loop. EL: please follow up. |  |
| 21 | [Mandatory] Power trace for VDD18, DVDD18 should be no less than 10mil Power trace for VDD33 should be no less than 5mil EL:ok |  |
| 22 | [Mandatory] - Stub on hi-speed trace should keep minim. E.g. If USB RX trace implement a pull-down resistor (Rb), usually 220K, then must not put a via on USB RX trace, the resistor must be very close to the trace, and ground point should be very close to resistor - USB RX traces shall not have any common-mode-choke, check schematic. EL:ok |  |
| 23 | ESD devices should normally put close to the connector, avoid via. [Mandatory] - ESD devices should have good GND connection. ESD GND connect to ground immediately. - ESD device shall not leave any Stub on RX/TX traces. EL: please add ESD components. |  |